# Best Management Practice Section and Design Job Aid: Sediment Control Fence

Construction stormwater Best Management Practice must be installed, inspected, maintained and removed in accordance with specific manufacturer specifications, where applicable, and the Construction General Permit (TXR150000).

## Description

A Sediment Control Fence (or silt fence) (SCF) is a temporary barrier made of woven geo-textile fabric that filters sediments and temporarily retains runoff before the runoff discharges from the project.

Figure 1. Best Management Practice (BMP) sediment control fence installed along a construction site to minimize erosion and prevent sediment from leaving the project area.

## Types

Temporary Sediment Control Fence (SCF) – a fence used in standard conditions

## Application

SCF is used to treat overland, non-concentrated flows. General use includes:

Along perimeter of project (if the roadway elevation is higher than the Right of Way (ROW) elevation)

Down-slope of exposed soil areas

Along streams and channels Around temporary stockpiles

Around areas of higher flow

Below the toe of exposed and erodible slopes

### Advantages

Cost effective sediment control device

Filters out sediment during rain events

Slows down stormwater runoff

### Disadvantages

Localized flooding due to minor ponding at the upslope side of the sediment control fence

Can fail structurally under heavy storm flows

Must be removed and disposed of

For projects using Amphibian and Reptile Exclusion Fence (AREF), contractors may confuse usage with silt fence

## Design Criteria

SCF placement must be parallel to slope contours along a line of constant elevation.

Should not be used in areas of concentrated flow.

A 2-year storm frequency may be used to calculate flow rate to be filtered.

SCF should be sized to filter a maximum flow rate through rate of 100 gallons per minute per square foot.

Not recommended to control erosion from a drainage larger than 2 acres. Additional controls may be needed.

Provide space for sediment removal activities and equipment between the SCF and other obstructions.

SCF is primarily used as a perimeter control on the down slope side of disturbed areas.

## References

Texas Department of Transportation (TxDOT) Temporary Erosion, Sediment and Water Pollution Control Measures: Fence & Vertical Tracking: EC (1) – 16

TxDOT Standard Specifications for Construction and Maintenance of Highways, Streets, and Bridges, November 1, 2014, Item 506, Temporary Erosion, Sedimentation, and Environmental Controls: (Sections 2.9 & 4.4.8)