# Best Management Practice Section and Design Job Aid: Tree Preservation

Construction stormwater Best Management Practice (BMP) must be installed, inspected, maintained and removed in accordance with Texas Department of Transportation (TxDOT) manufacturer specifications, where applicable, and the Construction General Permit (TXR150000).

## Description

Tree protection within the construction area is used to minimize erosion by protecting tree roots to allow for continued growth. Tree health and canopy density prevents splash erosion. Additionally, the tree roots provide natural soil stabilization.

Figure 1. Tree preservation is implemented around heritage trees by installing a perimeter fence and signage.

## Types

Orange Construction Fencing

Chain-Link Fence

Wood Barricade

## Application

Tree protection is utilized for permanent soil stabilization, run-on and runoff control, and temporary sediment control. Tree protection should be scheduled for sites with the following:

Healthy mature trees

Trees listed in the threatened and endangered plants list

Existing trees that help prevent erosion by increasing infiltration and decreasing stormwater velocity

Construction chemical use that will not affect existing vegetation

### Advantages

Most inexpensive form of erosion control

Most effective Best Management Practice (BMP) for soil stabilization and erosion control

Some trees may be transplanted within the construction sites

### Disadvantages

Site conditions will dictate design and need

Trees may need to be evaluated by an arborist

Onsite utilities may not allow the preservation of existing trees

## Design Criteria

- This BMP must be used in conjunction with construction activity scheduling and preservation of existing vegetation. Consider creating an inventory of trees onsite and identify trees that may provide added erosion and sediment control benefits.

- Identify trees that are to be protected and include the buffer on the Stormwater Pollution Prevention Plan (SWP3) Layout Sheets.

- If work is being conducted on an easement, confirm that the local government doesn’t have a tree protection ordinance.

- Preserve trees if they are on the threatened and endangered species plant list.

- On the SWP3 Layout Sheets, use the construction perimeter fence symbol to indicate the trees are to be preserved and a construction perimeter fence is needed to protect.

- The construction perimeter fence should be installed out from under the drip line of the tree to ensure adequate protection of the tree roots

- Workers, equipment, and vehicles should be situated away from existing trees to prevent trunk or root damage.

## References

Texas Pollutant Discharge Elimination System (TPDES) Construction General Permit TXR150000 Part III Section F and Part IV