# Best Management Practice Inspection Job Aid: Sediment Trap

This job aid is a tool to assist with the completion of Form 2118. All required corrective actions, comments, or discussions must be documented on Form 2118.

Review each criterion below. For each item, check the box or mark an “X” next to the appropriate response to indicate the compliance status: Yes (Y), No (N), or Not Applicable (NA) to indicate the compliance status.

Yes – The requirement has been met.

No – The requirement has not been met.

Not Applicable – The requirement does not apply to this project or site conditions.

## Installation criteria

- Have the sediment traps been documented in the Stormwater

- Discharge Map or Erosion Control Sheets?	☐Y   ☐N   ☐NA

- Are the sediment traps installed in accordance with the

- Stormwater Discharge Map or Erosion Control Sheets?	☐Y   ☐N   ☐NA

- Is the drainage area for the sediment trap less than 5 acres?	☐Y   ☐N   ☐NA

- Is the trap capacity 1800 cubic feet per acre?	☐Y   ☐N   ☐NA

- Were the sediment traps installed prior to construction and or

- land disturbing activities?	☐Y   ☐N   ☐NA

- Are side slopes within the safety clear zone of a roadway

- 6‑to‑1 (H:V) gradient or flatter?	☐Y   ☐N   ☐NA

- Are side slopes of the stone embankment 2‑to‑1 (H:V) gradient

- or flatter?	☐Y   ☐N   ☐NA

- Does the embankment have a max height of 5 feet?	☐Y   ☐N   ☐NA

- Does the embankment have a max top width of 4 feet?	☐Y   ☐N   ☐NA

- Has the storage area for a stone outlet sediment trap been

- constructed according to the limits of excavation as shown in

- the plans?	☐Y   ☐N   ☐NA

- Is the sandbag material composed of polypropylene, polyethylene,

- or polyamide woven fabric?	☐Y   ☐N   ☐NA

- Are the embankment stones used in accordance with specifications

- (for example, stone diameter, material, etcetera)?	☐Y   ☐N   ☐NA

## Inspection criteria

- Are the Best Management Practices (BMPs) functioning properly

- (for example, detaining runoff, trapping sediment, etcetera)?	☐Y   ☐N   ☐NA

- Has the trap capacity been reduced by 12 or has sediment

- accumulated to a depth of 1 foot? If yes, then clean out trap?	☐Y   ☐N   ☐NA

- Is the embankment clogged with sediments? If yes, then remove?	☐Y   ☐N   ☐NA

- Has efficiency of filter stone been decreased? If yes, then replace?	☐Y   ☐N   ☐NA

- Has trapped sediment been disposed of properly?	☐Y   ☐N   ☐NA

- Were any concerns identified with the stone outlet sediment

- traps?	☐Y   ☐N   ☐NA

- Were maintenance requirements recommended to the contractor?	☐Y   ☐N   ☐NA

## Maintenance criteria

- Were maintenance requirements recommended to the contractor?	☐Y   ☐N   ☐NA

- Are associated BMPs functioning properly?	☐Y   ☐N   ☐NA

- Is there any trash or debris inside the trap? If yes, then remove?	☐Y   ☐N   ☐NA

- Has accumulated sediment been removed and disposed of

- properly?	☐Y   ☐N   ☐NA

- Did the contractor address all recommended maintenance needs?	☐Y   ☐N   ☐NA

- Were any additional BMPs recommended to the Contractor?	☐Y   ☐N   ☐NA

Comments and Notes

[Enter comments and notes]

## Reference

Texas Pollutant Discharge Elimination System (TPDES) Construction General Permit TXR150000

Texas Department of Transportation (TxDOT) Temporary Erosion, Sediment, and Water Pollution Control Measures: Sediment Basins and Traps: EC (6) - 16